- An address in a block is given as 223.15.145.16. Find the number of addresses in the block, the first address, and the last address.

- Assume Class-C Address 200.25.49.5. Say the network is divided into 4 subnets. Calculate the subnet mask. List out the Subnet address and the first five addresses of each subnet.

- Find the error, if any, in the following IPv4 addresses:

a. 111.56.045.78

b. 221.34.7.8.20

c. 75.45.301.14

d. 11100010.23.14.67

Justify your answer for each of the given IP addresses.

- Convert the following IPv4 addresses from binary notation to dotted-decimal notation and identify the class of each address.

- 11110000 11000101 01010101 10101110

- 10100010 01110100 11001100 11001101.

- Perform bit-wise AND on 103.201.58.101 and 158.25.120.45.

- In a block of addresses, we know the IP address of one host is 182.44.82.16/26. Find out the network mask, first address (network address), last address (broadcast address), and the no. of valid host addresses in this block.

- Find the no. of subnet bits and the subnet mask in each case:

- 1024 subnets in class A

- 256 subnets in class B