- Give the network id and host id of the following IPv4 address: a) 56.89.34.25 b) 165.25.242.8 c)212.115.65.24 d) 227.54.68.26 e) 249.84.69.54

Ans. a) 56.89.34.25 Network id 56, Host id: 89.34.25 (CLASS A)

b) 165.25.242.8 Network id 165.25, Host id: 242.8 (CLASS B)

c)212.115.65.24 Network id 212.115.65, Host id: 24 (CLASS C)

d) 227.54.68.26 In Class D, an IP address is reserved for multicast addresses. It does not possess subnetting. Class D IP addresses are not allocated to hosts and are used for multicasting. Multicasting allows a single host to send a single stream of data to thousands of hosts across the Internet at the same time. It is often used for audio and video streaming, such as IP-based cable TV networks. Another example is the delivery of real-time stock market data from one source to many brokerage companies.

e) 249.84.69.54 This class is reserved for research and Development Purposes. (CLASS E)

- If a class B network on the Internet has a subnet mask of 255.255.240.0, what is the maximum number of hosts per subnet.

Ans. The 'subnet mask 255.255.240.0' has binary representation as, '11111111 11111111 11110000 00000000'. Here in this '20 bits' set in the subnet. So (32-20) = 12 bits are taken for host ids.

Hence, the 'Total possible values of host ids' are '2^12 = ‘4096'. Out of these '4096' values, two 'addresses' are reserved. The 'address' with all 'bits as 1' is reserved as 'broadcast address' and 'address' with all 'host' id bits as '0' is used as the subnet network address.

The number of 'addresses' usable for 'addressing' specific 'hosts in each network' is always '2^N - 2' where N is the 'number of bits for host id'.

In our 'case for subnet mask 255.255.240.0', number of hosts is '2^N - 2' = '2^12 - 2' = '4096 - 2 = 4094'.

- Given an IP address 211.25.38.26. Find the network ID,5th host ID, last host ID and broadcast ID of the given network.

Ans. 211.25.38.26, It is belonging to class C and its n value is 24.

The binary representation of the given address is.

11010011	00011001	00100110	00011010

The first address in the block can be found by setting the rightmost 32 − n bits to 0s.

If we set 32−24 rightmost bits to 0, we get,

11010011	00011001	00100110	00000000 or 211.25.38.0

The last address (broadcast ID) in the block can be found by setting the rightmost 32 − n bits to 1s.

If we set 32−24 rightmost bits to 1, we get,

11010011	00011001	00100110	11111111 or 211.25.38.255

5th host ID, 211.25.38.5

last host ID, 211.25.38.254

network ID, 211.25.38.0

broadcast ID, 211.25.38.255

- Using fixed length subnetting divide the given network 196.220.84.32 into two parts. Give the first host ID, Last host ID, Subnet mask and the number of hosts connected.

Ans. Clearly, the given network belongs to class C.

We have a big single network having IP Address 196.220.84.32

Network mask is 255.255.255.0

Binary representation is 11000100 11011100 01010100 00100000

For creating two subnets and to represent their subnet IDs, we require 1 bit. So, we borrow one bit from the Host ID part. After borrowing one bit, Host ID part remains with only 7 bits.

11000100.11011100.01010100.0/1 (host id)

If borrowed bit = 0, then it represents the first subnet.

If borrowed bit = 1, then it represents the second subnet.

IP Address of the two subnets are-

196.220.84.00000000 = 196.220.84.0

196.220.84.10000000 = 196.220.84.128

For 1st Subnet-

First subnet n1=25

IP Address of the subnet1 = 196.220.84.0/25

Total number of IP Addresses = 2^7 = 128

Total number of hosts that can be configured = 128 – 2 = 126

Range of IP Addresses = [196.220.84.00000000, 196.220.84.01111111] = [196.220.84.0, 196.220.84.127]

Direct Broadcast Address = 196.220.84.01111111 = 196.220.84.127

Subnet1 mask = 255.255.255.128

first host ID: 196.220.84.1

Last host ID: 196.220.84.126

For 2nd Subnet-

First subnet n2=25

IP Address of the subnet2 = 196.220.84.128/25

Total number of IP Addresses = 2^7 = 128

Total number of hosts that can be configured = 128 – 2 = 126

Range of IP Addresses = [196.220.84.10000000, 196.220.84.11111111] = [196.220.84.128, 196.220.84.125]

Direct Broadcast Address = 196.220.84.11111111 = 196.220.84.255

Subnet2 mask = 255.255.255.128

first host ID: 196.220.84.129

Last host ID: 196.220.84.254

- Find the network address, first host address, last host address, default mask for the following ip address 1)110.56.28.55 2) 142.120.48.23 3) 206.156.36.84

Ans. 1)110.56.28.55 it is coming under CLASS A, DEFAULT MASK 255.0.0.0, first host address is 110.0.0.1, last host address 110.255.255.254, network address 110.0.0.0

2) 142.120.48.23 it is coming under CLASS B, DEFAULT MASK 255.255.0.0, first host address is 142.120.0.1, last host address 142.120.255.254, network address 142.120.0.0

3) 206.156.36.84 it is coming under CLASS C, DEFAULT MASK 255.255.255.0, first host address is 206.156.36.1, last host address 206.156.36.254, network address 206.156.36.0

- Divide the network using variable length subnetting (2 subnets) for the network 192.10.25.130/26 give the first host ID, last host ID, Subnet mask and network address.

Ans. We have a big single network having IP Address 192.10.25.130

Net mask is 255.255.255.192

Binary representation is 11000000 00001010 00011001 10000010

For creating two subnets and to represent their subnet IDs, we require 1 bit.

So, we borrow one bit from the Host ID part.

After borrowing one bit, Host ID part remains with only 5 bits.

11000000 00001010 00011001 10 0/1 (host id)

If borrowed bit = 0, then it represents the first subnet.

If borrowed bit = 1, then it represents the second subnet.

Network (IP) Address of the two subnets is-

192.10.25.10000000 = 192.10.25.128

192.10.25.10100000 = 192.10.25.160

For 1st Subnet-

First subnet n1=27

IP Address of the subnet = 192.10.25.128/27

Total number of IP Addresses = 2^5 = 32

Total number of hosts that can be configured = 32 – 2 = 30

Range of IP Addresses = [192.10.25.10000000, 192.10.25.10011111] = [192.10.25.128, 192.10.25.159]

Direct Broadcast Address = 192.10.25.10011111= 192.10.25.159

Subnet1 mask= 255.255.255.224

first host ID: 192.10.25.129

Last host ID: 192.10.25.158

For 2nd Subnet-

Second subnet n2=27

IP Address of the subnet = 192.10.25.160/27

Total number of IP Addresses = 2^5 = 32

Total number of hosts that can be configured = 32 – 2 = 30

Range of IP Addresses = [192.10.25.10100000, 192.10.25.10111111] = [192.10.25.160, 192.10.25.191]

Direct Broadcast Address = 192.10.25.10111111= 192.10.25.191

Subnet2 mask=255.255.255.224

first host ID: 192.10.25.161

Last host ID: 192.10.25.190

- Given an IP address 192.168.12.56 Give the following. 1. Network address 2. Default mask 3.1st host ID 4. Last host ID 5. Broadcast address.

Ans. Given IP Address is 192. 168.12.56

Binary representation is 11000000 10101000 00001100 00111000

Total number of IP Addresses possible are 2^8 = 256

Total number of hosts that can be configured = 256– 2 = 254

Network address: 192.168.12.0

Default mask: 255.255.255.0

1st host ID: 192.168.12.0

Last host ID: 192.168.12.254

Broadcast address: 192.168.12.255

- Given the IP address 192.151.15.10. If the network is divided into 4 subnets. Calculate the subnet mask and range of addresses for every subnet.

- Given IP address as 121.30.28.11. Find the Class and network mask of the given IP address and write the above IP address in Binary format.

- An address in a block is given as 169.121.16.35. Compute the following: i) Class of IP and Network Mask. ii) First Address (Network address) of a block. iii) Last address (Broadcast address) of a block. iv) Total no. of addresses in a block. v) Number of Valid host addresses in a block.

- Apply the slash notation (/n) over the below IP addresses and rewrite them in slash notation. A. 255.0.0.0 B. 255.255.224.0 C. 255.255.240.0 D.255.240.0.0

- Find the error, if there are any, in the following IPv4 addresses. (a) 111.56.045.78 (b) 221.34.7.8.20 (c) 75.45.301.14 (d) 11100010.23.14.67

- Convert the given IPV6 address into its short notation. FDEC: 0074: 0000: 0000: 0000: B0FF: 0000: FFF0

- Leading Zeros: Leading zeros in each 16-bit hexadecimal group can be omitted.

- Consecutive Zero Groups: One or more consecutive groups of all zeros can be replaced with a double colon (::). This can only be done once in an address.

Applying the Rules:

- Remove Leading Zeros:

  - FDEC:0074:0000:0000:0000:B0FF:0000:FFF0 becomes

  - FDEC:74:0:0:0:B0FF:0:FFF0

- Replace Consecutive Zero Groups:

  - The three consecutive zero groups (0:0:0) can be replaced with ::.

  - FDEC:74::B0FF:0:FFF0

- Remove the remaining leading zero:

  - FDEC:74::B0FF:0:FFF0 becomes

  - FDEC:74::B0FF:0:FFF0

- Change the following IPv4 addresses from binary notation to dotted-decimal notation. 10000001 00001011 01001011 11101111

- A network with pure ALOHA transmits 250 bits per frame on a shared channel of 500 kbps. Compute the time required to make the transmission collision free?

Ans. Average Frame Transmission time Tfr is 250 bits/500 kbps or 0.5 ms.

The vulnerable time is 2 X 0.5ms= 1ms.

This means no station should send later than 0.5 ms before this station starts transmission and no station should start sending during the one 0.5- ms period that this station is sending.