ELECTROMAGNETIC FIELD THEORY

VECTOR ANALYSIS

Vector analysis is mathematical shorthand. It has some new symbols, some new rules, and a pitfall here and there like most new fields, and it demands concentration, attention, and practice. The fundamental concepts and operations are covered in this chapter.

SCALAR

The term scalar refers to a quantity whose value may be represented  by a single (positive or negative) real number.

If we speak of a body falling a distance  L in a time t, or the temperature T  at any point  in a bowl of soup whose coordinates are x, y, and z, then L, t, T , x, y, and z are all scalars. Other scalar quantities are mass, density, pressure (but not force), volume, and volume resistivity. Voltage is also a scalar quantity, although  the complex representation of a sinusoidal  voltage,  an  artificial  procedure,  produces  a complex scalar,  or phasor, which requires two real numbers for its representation, such as amplitude and phase angle, or real part  and imaginary  part.

VECTOR

A vector quantity has both a magnitude  and a direction in space. We shall be concerned with two- and three-dimensional spaces only, but vectors may be defined in n-dimensional space in more advanced applications.

Force, velocity, acceleration, and a straight line from the positive to the negative terminal of a storage battery are examples of vectors. Each quantity is characterized by both a magnitude and a direction.

VECTOR ALGEBRA

Vector algebra  has its own set of rules, we now proceed  to  define the rules of vector  arithmetic,  vector  algebra,  and of vector calculus. Some of the rules will be similar to those of scalar algebra, some will differ slightly, and some will be entirely new and strange.

VECTOR ADDITION

Vector addition follows the parallelogram law, and this can be accomplished graphically as shown in Fig.1

FIGURE.1

Two  vectors  may  be  added  graphically  either  by  drawing  both  vectors  from  a  common  origin  and completing the parallelogram or by beginning the second vector from the head of the first and completing the triangle; either method  is easily extended to three or more vectors.

Vector addition obeys the commutative and associative laws

i.e       A + B = B + A	commutative law

and     A + (B + C) = (A + B)+ C   associative law.

Coplanar vectors, or vectors lying in a common plane, such as those shown in Fig.1 may  also  be added  by expressing  each  vector  in  terms  of  “horizontal''  and  “vertical'' components and adding  the corresponding components.

SUBTRACTION

The rule for the subtraction of vectors follows easily from that for addition, for  we may  always  express  A - B as A + (-B);  the  sign, or  direction,  of the second vector is reversed, and  this vector is then added  to the first by the rule for vector addition.

Two  vectors  are  said  to  be equal  if their  difference  is zero,  or  A = B if  A - B =O.

MULTIPLICATION

Multiplication involving vectors will be of the following types

1. Multiplying a vector by scalar

2. Multiplying a vector by another vector

When a vector is multiplied by a scalar the magnitude of the vector changes, but its direction does not when the scalar is positive, although it reverses direction when multiplied by a negative scalar. Multiplication of a vector by a scalar also obeys the associative and distributive laws of algebra, leading to

(r + s)(A + B) = r(A + B)+ s(A + B) = rA + rB + sA + sB

The discussion on multiplying a vector by another vector is postponed for the completion of rectangular coordinate system.

DIVISION

Division of a vector by a scalar is merely multiplication by the reciprocal of that  scalar.

COORDINATE SYSTEMS

In order to describe a vector accurately, some specific lengths, directions, angles, projections,  or components  must  be given. There  are three  simple methods  of doing this, and about  eight or ten other methods which are useful in very special cases. We are going to use only the three  simple methods,

1.Cartesian or rectangular coordinate system.

2.Circular cylindrical coordinate system

3.Spherical coordinate system

1. THE CARTESIAN COORDINATE SYSTEM

In the Cartesian coordinate  system we set up three coordinate  axes mutually at right angles to each other, and call them the x, y, and z axes. It is customary to choose a right-handed coordinate  system, in which a rotation (through  the smaller angle)  of the  x axis into  the  y axis would  cause  a  right-handed screw to progress  in  the  direction  of  the  z  axis. Fig.2a shows a right-handed Cartesian  coordinate  system.

FIGURE.2a A  right-handed cartesian  coordinate   system.  If  the  curved  fingers  of  the  right  hand  indicate  the direction through  which the x axis is turned into coincidence with the y axis, the thumb shows the direction of the z axis.

POINT

A point in the space is defined by giving its x, y, and z coordinates. These are, respectively,  the  distances  from  the  origin  to  the  intersection   of  a  perpendicular dropped  from the point  to the x, y, and z axes.

An alternative  method  of interpreting  coordinate  values ,  is to  consider  the  point  as being at  the common  intersection  of three  surfaces,  the planes  x = constant,  y = constant, and z = constant,  the constants  being the coordinate  values of the point.Fig.2b shows  the  points  P  and  Q whose  coordinates are  (1, 2, 3) and (2, -2, 1),  respectively.  Point  P  is therefore  located  at  the  common  point  of intersection  of the planes  x = 1, y = 2, and  z = 3, while point  Q is located  at the intersection  of the planes x = 2, y = -2, z = 1.

FIGURE.2b The location of points P(l, 2, 3) and Q(2, -2, l).

In the other two coordinate systems, a point in the space is visualized as the common  intersection  of three  surfaces,  not necessarily planes, but still mutually  perpendicular at the point  of intersection.

VECTOR COMPONENTS AND UNIT VECTORS

To describe a vector  in the cartesian  coordinate  system, let us first consider  a vector r extending outward  from the origin. A logical way to identify this vector is by giving the three component vectors, lying along the three coordinate  axes, whose  vector  sum  must  be the  given vector.  If the  component vectors  of the vector r are x, y, and z, then r = x + y + z. The component vectors are shown in Fig.3a. Instead  of one vector, we now have three,  but this is a step forward, because  the three  vectors  are  of a very simple nature;  each is always directed along one of the coordinate  axes.

FIGURE.3a The component vectors x, y, and z of vector r.

The component vectors have magnitudes which depend on the given vector (such as r above),  but  they each have a known  and  constant direction.  This suggests the use of unit vectors having unit magnitude, by definition, and directed along the coordinate axes in the direction  of the increasing coordinate  values. We shall reserve the symbol a for  unit vector and identify the direction  of the unit vector by an appropriate subscript.

Thus ax , ay , and az are the unit vectors in the cartesian  coordinate system. They are directed along the x, y, and z axes, respectively, as shown in Fig.3b.

FIGURE.3b The unit vectors of the cartesian  coordinate system have unit magnitude  and are directed toward  increasing values of their respective variables.

POSITION VECTOR

The vector r extending outward  from the origin to the point (x, y, z) is called as the position vector which can be expressed mathematically as

r = x ax + y ay + z az

A vector rP  pointing  from the origin to point P(l, 2, 3) is written rP  = ax   + 2ay + 3az . The vector from P to Q may be obtained by applying the rule of vector addition.  This rule shows that  the vector from the origin to P plus the vector from P to Q is equal to the vector from the origin to Q. The desired vector from  P(l, 2, 3) to Q(2, -2, l) is therefore

RPQ   = rQ  - rP  = (2 - l) ax   + (-2 - 2) ay   + (l - 3) az

= ax - 4ay - 2az

The vectors rP , rQ , and RPQ   are shown in Fig.3c.

FIGURE.3c The vector RPQ   is equal to the vector difference rQ  - rP .

Any vector A then may be described by A = Ax ax + Ay ay + Az az. The magnitude  of A written  |A| or simply A, is given by

|A| = ( Ax 2 + Ay 2 + Az 2)1/2

Each of the three coordinate systems will have its three fundamental and  mutually  perpendicular unit  vectors which are used to resolve any vector into its component vectors, also to calculate the unit vector along any vector B we write aB = B/|B|

DIFFERENTIAL ELEMENTS

In rectangular coordinate system the differential elements, volume, surface and length can be expressed mathematically as follows.

If we visualize three planes intersecting at the general point P, whose coordinates  are x, y, and z, we may increase each coordinate  value by a differential amount  and obtain  three slightly displaced planes intersecting at point P1, whose coordinates are x + dx, y + dy, and  z + dz. The six planes define a rectangular parallelepiped as shown in the fig.4 .The point  P1  is indicated,  but  point  P  is located  at the  invisible corner.

FIGURE.4 The differential  volume element in Cartesian coordinates; dx, dy and dz are in general, independent  differentials.

Now the differential volume dv = dxdydz (scalar)

the differential length,          dL = dx ax + dy ay + dz az ( vector)

and the six differential surfaces are  ±dxdy az , ±dxdz ay, ±dydz ax (vectors)

THE FIELD

A field  (scalar or vector) may be defined mathematically as some function  of that vector which connects  an  arbitrary origin  to  a  general  point  in  space or simply a field is the  function of a position vector. If r is the position vector then a field F can be expressed in functional notation as F(r).

## Scalar Field

## A scalar field is a function that gives us a single value of some variable for every point in space (or) A scalar field is a field for which there is a single number associated with every point in space.

As an example, let the scalar field F be defined as

F(r)  = F(x,y,z) =  xy – x2yz + yz

This expression defines the value of the scalar function F at every point (x,y,z) in space.

VECTOR Field

A vector field is a quantity which has both a magnitude and a direction at every point in space. Vectors fields  are used to describe physical quantities such as velocity, momentum, acceleration and force, associated with an object.

As an example, let the vector field F be defined as

F(r)  = F(x,y,z) = xy ax – x2yz ay  + yz az

This expression defines the value of the vector function F at every point (x,y,z) in space.

PRODUCT OF TWO VECTORS

Let us now consider multiplication of a vector by another vector which is of two types, one is the dot product and the other cross product.

THE DOT PRODUCT

Given two vectors A and B, the dot product, or scalar product, is defined as the product  of the magnitude  of A, the magnitude  of B, and  the cosine of the smaller angle between them,

A·B = |A| |B| cos θAB

The dot, or scalar, product is a scalar, as one of the names implies, and  it obeys the commutative law,

A·B  = B·A

APPLICATIONS

1. The most common application of the dot product is in mechanics, where a constant force F applied over a straight displacement L does an amount of work FL cos θ, which can be more easily written as F·L

If the force F varies along the path, then the total work done W = ∫ F·dL

2. The total flux crossing a surface of area S is given by BS if the magnetic flux density B is perpendicular to the surface and uniform over it.

If we define a vector surface S as having the usual area for its magnitude and having a direction normal to the surface and the magnetic flux density B is uniform over S, then flux crossing the surface is B ·S.

If the flux density is not constant over the surface, then the total flux, ϕ = ∫ B·dS

3. The angle between two vectors in three-dimensional space can be calculated as follows

If A = Ax ax + Ay ay + Az az and  B = Bx ax + By ay + Bz az then cosθ = A·B ̸ |A| |B|

4. One of the most important applications of the dot product is that of finding the component of a vector in a given direction.

FIGURE 5

Fig5(a) shows the scalar component of B in the direction of the unit vector a which is B·a. and (b) shows the vector component of B in the direction  of the unit vector a as (B·a)a. The scalar component B·a is also called as projection of B along the direction a.

THE CROSS PRODUCT

Given two vectors A and B, the cross product or vector product of A and B is written as A x B.The cross product  A x B is a vector; the magnitude of A x B is equal to the product  of the magnitudes  of A, B and the sine of the smaller angle between A and B; the direction of A x B is perpendicular to the plane containing A and B and is along that one of the two possible perpendiculars which is in the direction of advance of a right-handed screw as A is turned into B. This direction is illustrated  in Fig.6

A x B = aN |A| |B| sin θAB

The direction of A x B is in the direction of advance of a right-handed screw as A is turned  into B.

The cross product  is not  commutative, for B x A = -(A x B).

If the definition  of the cross product  is applied  to the unit vectors, ax x ay =  az ; ay x az =  ax and az x ax = ay .

FIGURE 6

APPLICATIONS

1. To find the area of a parallelogram, the product  of the lengths of two adjacent  sides is multiplied  by the sine of the angle between them. Using vector notation for the two sides, we then may express the (scalar) area as the magnitude of A x B, or |A x B|.

2.Consider  the  force  on  a  straight  conductor of  length  L, where  the  direction  assigned  to  L  corresponds  to  the  direction  of the  steady current I , and a uniform magnetic field of flux density B is present. Using vector notation, we may write the result neatly as F = I L x B.

CALCULATION OF AxB

The evaluation  of a cross product  by means of its definition turns out to be more  work  than  the evaluation  of the dot  product  from  its definition,  for not only must we find the angle between the vectors, but we must find an expression for the unit vector aN . This work may be avoided by using cartesian components for the two vectors A and B and expanding  the cross product  as a sum of nine simpler cross products,

A x B = Ax Bx ax x ax   + Ax By ax x ay + Ax  Bz axx az

+ Ay Bx ayx ax  + Ay By ayx ay + Ay Bz ayx az

+ Az Bx az x ax  + Az By az x ay + Az Bz az x az

since ax x ay  =  az ; ay  x az  =  ax and az  x ax  = ay and the cross product  of any vector with itself is zero,

A x B = (Ay Bz –Az By )ax + (Az Bx –Ax Bz )ay +(Ax By –Ay Bx )az

2. CIRCULAR CYLINDRICAL COORDINATE SYSTEM

The Cartesian  coordinate  system is generally the one in which students  prefer to work every problem.  This often means a lot more work for the student,  because many  problems  possess  a  type  of symmetry  which  pleads  for  a  more  logical treatment. It is easier to do now, the work required  to become familiar with cylindrical and spherical coordinates, instead of applying an equal or greater  effort  to  every problem  involving  cylindrical  or spherical  symmetry later.  With  this  future  saving  of  labour  in  mind,  we shall  take  a  careful  and unhurried look at cylindrical and spherical coordinates.

The circular cylindrical coordinate  system is the three-dimensional version of  the  polar  coordinates of  analytic  geometry.  In  the  two-dimensional polar coordinates, a point  was located  in a plane  by giving its distance  ρ from  the origin,  and  the  angle  ϕ between  the  line from  the  point  to  the  origin  and  an arbitrary radial  line, taken  as ϕ= 0.  A three-dimensional coordinate  system, circular  cylindrical coordinates, is obtained  by also specifying the distance  z of the point  from an arbitrary z = 0 reference plane which is perpendicular to the line ρ = 0. For  simplicity,  we usually  refer  to  circular  cylindrical  coordinates simply as cylindrical coordinates, but it is fair to point  out that  there are such systems as elliptic cylindrical coordinates, hyperbolic  cylindrical coordinates, parabolic  cylindrical coordinates, and others.

FIGURE 7(a)The three mutually perpendicular surfaces of the circular cylindrical coordinate system.

POINT

We no longer set up three axes as in cartesian coordinates, but must instead consider  any point  as the intersection  of three mutually  perpendicular surfaces. These surfaces are a circular cylinder (ρ = constant), a plane (ϕ = constant), and another  plane  (z = constant) as shown in fig.7(a).Note  that  three  such  surfaces  may  be  passed through  any point,  unless it lies on the z axis, in which case one plane suffices.

UNIT VECTORS

Three unit vectors must also be defined, but we may no longer direct them along  the "coordinate axes,''  for such axes exist only in cartesian  coordinates. Instead,  we take a broader  view of the unit vectors in cartesian  coordinates and realize that  they are directed  toward  increasing  coordinate  values and  are perpendicular to the surface on which that coordinate  value is constant  (i.e., the unit vector ax  is normal  to the plane x = constant  and points toward  larger values of x). In a corresponding way we may now define three unit vectors in cylindrical coordinates, aρ , aϕ , and az .

FIGURE 7(b)The three unit vectors of the circular cylindrical coordinate system.

The  unit  vector  aρ at  a  point  P(ρ1 , ϕ1 , z1 ) is directed  radially  outward, normal   to  the  cylindrical  surface  ρ= ρ1 .  It  lies  in  the  planes  ϕ = ϕ1     and z = z1 .

The unit vector aϕ is normal  to the plane ϕ= ϕ1 , points in the direction of increasing ϕ, lies in the plane z = z1 , and is tangent  to the cylindrical surface ρ= ρ1.

The  unit  vector  az is the  same  as the  unit  vector  az of the  cartesian coordinate  system. Fig.7(b) shows the three vectors in cylindrical coordinates.

In cartesian  coordinates, the unit  vectors are not  functions  of the coordinates. Two of the unit vectors in cylindrical coordinates, aρ and aϕ, do vary with the coordinate ϕ, since their directions  change.  Hence, in integration or differentiation with respect to ϕ, aρ and aϕ  must not be treated  as constants.

The unit  vectors are again  mutually  perpendicular, for each is normal  to one  of the three  mutually  perpendicular surfaces,  and  we  define a right- handed  cylindrical coordinate  system as one in which aρ x aϕ  = az ,aϕ x az  = aρ and az x aρ = aϕ .

Any vector A then may be described by A = Aρ aρ   + Aϕ aϕ + Az az  i.e. by identifying the components of A along the unit vectors the vector A can be described completely.

POSITION VECTOR

The vector r extending from origin to any point (ρ,ϕ,z)  is called as position vector. The components of r along the unit vectors aρ , aϕ, az are ρ,0,z respectively. Hence

r = ρ aρ + z az is the position vector of the point (ρ,ϕ,z).  DIFFERENTIAL ELEMENTS

A differential  volume element in cylindrical coordinates may be obtained by increasing ρ, ϕ,and z by the differential  increments  dρ, dϕ, and dz. The two cylinders of radius  ρ and ρ + dρ, the two radial  planes at angles ϕ and ϕ+ dϕ and the two "horizontal'' planes at "elevations'' z and z + dz now enclose a small volume,  as shown  in Fig.7(c), having  the shape  of a truncated wedge. As the volume element becomes very small, its shape approaches that  of a rectangular parallelepiped  having  sides of length dρ, ρdϕ  and  dz. Note  that  dρ and  dz are dimensionally lengths, but dϕ  is not; ρdϕ   is the length.

Now the differential volume dv = ρ dρ dϕ dz

the differential length,          dL = dρ aρ + ρdϕ   aϕ+ ρdϕ   az

and the six differential surfaces are  ± ρ dρ dϕ az, ± ρdϕ dz aρ, and ± dρ dz aϕ

RELATING RECTANGULAR AND CYLINDRICAL COORDINATES

The  variables  of  the  rectangular and  cylindrical  coordinate   systems can be related easily  to each other as shown in the figure given below.

FIGURE 8.The relationship between the cartesian variables x, y, z and the cylindrical coordinate  variables ρ , ϕ, z. There is no change in the variable z between the two systems

From the fig.8 we have

x = ρ cosϕ

y = ρ sinϕ

z = z

From  the other  viewpoint,  we may express the cylindrical variables in terms of x, y, and z as

ρ = (x2 + y 2 )1/2

ϕ = tan-1(y/x)

z = z

Here we shall  consider  the  variable  ρ to  be  positive  or  zero and the proper value of the angle ϕ is determined by inspecting  the signs of x and y. For example, if x = -3 and y = 4, we find that  the point  lies in the second quadrant so that  ρ = 5 and ϕ = 126.9°. For  x = 3  and  y = -4, we have ϕ = -53.1° or 306.9°, whichever is more convenient.

From the above set of equations, scalar  functions  given in  one  coordinate   system can be  easily transformed into the other  system.

VECTOR TRANSFORMATION : RECTANGULAR <=> CYLINDRICAL

A vector function  in one coordinate  system,  requires two steps in order  to  transform it to  another  coordinate  system

RECTANGULAR TO CYLINDRICAL

Let A be the vector in rectangular coordinates

A = Ax ax  + Ay ay  + Az az , here each component ( Ax, Ay, or Az) is a function of x, y, and z. After transformation the vector A should be in cylindrical coordinates and A = Aρ aρ + Aϕ aϕ + Az az , where each component is a function of   ρ, ϕ and z. This transformation requires two steps as said earlier.

Step1:Calculation of cylindrical components Aρ , Aϕ and Az (transformation of the components)

Since Aρ, Aϕ and Az are the components of given vector A (A = Ax ax  + Ay ay  + Az az ) along aρ, aϕ and az directions, Aρ= A·aρ  and Aϕ = A·aϕ ( as z coordinate is same in both the coordinate systems Az is also same and need not be calculated separately)

hence

Aρ= A·aρ = (Ax ax  + Ay ay  + Az az) ·aρ = Ax ax·aρ + Ay ay·aρ 	 (as az·aρ =0)

Aϕ = A·aϕ = (Ax ax  + Ay ay  + Az az) ·aϕ = Ax ax·aϕ + Ay ay·aϕ 	 (as az·aϕ =0)

also	Az= A·az  = (Ax ax  + Ay ay  + Az az) ·az  = Az az·az =  Az

Now it is necessary to know the  dot products  ax·aρ, ay·aρ, ax·aϕ, and ay·aϕ. Applying the definition of the dot product, from the fig.7(b) we have the values of the dot products as shown in the table.

|  | aρ | aϕ | az |
| --- | --- | --- | --- |
| ax · | cosϕ | -sinϕ | 0 |
| ay · | sinϕ | cosϕ | 0 |
| az · | 0 | 0 | 1 |

TABLE.1Dot products of unit vectors in cylindrical and cartesian coordinate systems

The values of the dot products can be used in the above expressions to calculate Aρ, Aϕ and Az

Step 2:In order to complete the transformation, Ax,  Ay and Az that appear in Aρ, Aϕ and Az are now to be expressed in terms of ρ, ϕ and z (because Ax,  Ay and Az are functions of x, y and z) using

x = ρ cosϕ

y = ρ sinϕ

z = z.

CYLINDRICAL TO RECTANGULAR

In order to transform a vector from cylindrical to rectangular coordinates, the procedure is identical to the one illustrated above.

3. THE  SPHERICAL COORDINATE SYSTEM

Let us start by building a spherical coordinate  system on the three cartesian axes . We first define the distance  from the origin to any point  as r. The second coordinate  is an angle θ between the z axis and the line drawn from the origin to the point.

The third coordinate  ϕ is also an angle and is exactly the same as the angle ϕ of cylindrical coordinates. It is the angle between the x axis and the projection in the z = 0 plane of the line drawn from the origin to the point.

FIGURE 9(a).The three spherical coordinates.

POINT

Here, we consider any point in the space as as the intersection  of three  mutually perpendicular surfaces as  shown in fig.9(b).

The three surfaces are

r = constant, a sphere

θ = constant, a cone and

ϕ = constant, a plane

The  two  surfaces,  cone  and  sphere,  are  everywhere  perpendicular along  their intersection,  which is a circle of radius  r sinθ.

FIGURE 9(b).The three mutually perpendicular surfaces of the spherical coordinate system.

UNIT VECTORS

Three unit vectors namely ar, aθ and aϕ can be defined at any point in space.  Each unit vector is perpendicular to one of the three mutually perpendicular surfaces and oriented in that  direction  in which the coordinate  increases.

FIGURE 9(c).The three unit vectors of spherical coordinates

The unit  vector  ar is directed radially  outward, normal  to  the sphere  r = constant,  and  lies in the cone θ = constant  and the plane ϕ = constant.

The unit vector aθ is normal  to the conical surface, lies in the plane, and is tangent to the sphere.

The  third  unit  vector  aϕ is the  same  as  in cylindrical  coordinates, being  normal  to  the  plane  and  tangent  to  both  the cone and sphere. The three unit vectors are mutually perpendicular.

Usually right- handed coordinate systems are used. We define the right-handed spherical coordinate systems as the one, in which ar x aθ = aϕ ;aθ x aϕ = ar  and aϕ x ar = aθ.

(Note: The system is said to be right handed if the rotation from ar to aθ through the smaller angle causes a right handed screw to progress in the direction of aϕ i.e. if  ar x aθ = aϕ )

Any vector A then may be described by A = Ar ar  + Aθ aθ + Aϕ aϕ i.e. by identifying the components of A along the unit vectors the vector A can be described completely.

POSITION VECTOR

The vector r extending from origin to any point (r, θ, ϕ)  is called as position vector. The components of r along the unit vectors ar, aθ and aϕ are r, 0  and 0 respectively. Hence

r = r ar  is the position vector of the point (r, θ, ϕ)

DIFFERENTIAL ELEMENTS

Differential  volume  element  may  be  constructed in  spherical  coordinates  by increasing  r, θ and  ϕ by dr, dθ and  dϕ , as shown  in Fig.

The distance between  the  two  spherical  surfaces  of  radius  r  and  r + dr  is dr;  the  distance between  the  two  cones  having  generating  angles  of θ and  θ + dθ    is r dθ ;  and the distance  between the two radial  planes  at angles ϕ and  ϕ + dϕ is found  to be r sinθ dϕ

FIGURE 9(d).The differential volume element in the spherical coordinate  system.

Now the differential volume dv = r2 sinθ dr dθ dϕ

the differential length,          dL = dr ar  + r dθ  aθ  + r sinθ dϕ aϕ

and the six differential surfaces are  ± r dr  dθ aϕ , ± r2 sinθ dθ dϕ ar  and  ± r sinθ dr dϕ aθ

RELATING RECTANGULAR AND SPHERICAL COORDINATES

The  variables  of  the  rectangular and  spherical  coordinate   systems can be related easily  to each other by inspecting  the figure given below.

x = r sinθ cosϕ

y = r sinθ sinϕ

z = r cosθ

and the transformation in the reverse direction  is achieved with the help of the following

r = (x2 + y2 + z2)1/2		(r ≥ 0)

θ = cos-1 (z√x2 + y2 + z2)		(00 ≤ θ ≤1800)

FIGURE 10			          ϕ = tan-1yx

The angles are placed in the proper  quadrants by inspecting the signs of x, y, and z.

VECTOR TRANSFORMATION: RECTANGULAR <=> SPHERICAL

In order to transform a vector, either from rectangular to spherical or from spherical to rectangular, it requires two steps.

RECTANGULAR TO SPHERICAL

Let A be the vector in rectangular coordinates

A = Ax ax  + Ay ay  + Az az , here each component ( Ax, Ay, Az ) is a function of x, y, and z. After transformation the vector A should be in spherical coordinates and A = Ar ar  + Aθ aθ + Aϕ aϕ , where each component is a function of   r, θ, and  ϕ. This transformation requires two steps as said earlier.

Step1:Calculation of spherical components Ar , Aθ and Aϕ (transformation of the components)

Since Ar , Aθ and Aϕ are the components of given vector A (A = Ax ax  + Ay ay  + Az az) along ar, aθ and aϕ directions, Ar = A·ar ;  Aθ = A·aθ and Aϕ = A·aϕ

hence

Ar = A·ar = (Ax ax  + Ay ay  + Az az ) ·ar = Ax ax·ar + Ay ay·ar + Az az·ar

Aθ = A·aθ = (Ax ax  + Ay ay  + Az az ) ·aθ = Ax ax·aθ + Ay ay·aθ + Az az·aθ

also	Aϕ = A·aϕ = (Ax ax  + Ay ay  + Az az ) ·aϕ = Ax ax·aϕ + Ay ay·aϕ + Az az·aϕ

Now it is necessary to know the  dot products of the unit vectors specified above. Since the dot  product  of any spherical  unit  vector  with  any  cartesian  unit  vector  is the  component of  the spherical  vector  in the direction  of the cartesian  vector,  the dot  products  with az  are found  to be

az·ar  =  cosθ

az·aθ =  -sinθ

az·aϕ =  0

The  dot  products  involving  ax and  ay require  first  the  projection  of the spherical  unit  vector on the  xy plane  and  then  the projection  onto  the desired axis.

For  example, ar· ax is obtained  by projecting  ar onto  the xy plane, giving  sinθ, and then projecting  sinθ on the x axis, which yields sinθ cosϕ. The other dot products  are found  in a similar FIGURE 11.

manner,  and all are shown in Table 2.

|  | ar | aθ | aϕ |
| --- | --- | --- | --- |
| ax · | sinθ cosϕ | cosθ cosϕ | -sinϕ |
| ay · | sinθ sinϕ | cosθ sinϕ | cosϕ |
| az · | cosθ | -sinθ | 0 |

TABLE.2Dot products of unit vectors in spherical and cartesian coordinate systems

Hence the values of the dot products can be used in the above expressions to calculate Ar , Aθ, and Aϕ .

Step 2:In order to complete the transformation, Ax ,  Ay and Az that appear in Ar , Aθ, and Aϕ are now to be expressed in terms of r, θ, and  ϕ (because Ax ,  Ay and Az are functions of x, y, and z) using

x = r sinθ cosϕ

y = r sinθ sinϕ

z = r cosθ

SPHERICAL TO RECTANGULAR

In order to transform a vector from spherical to rectangular coordinates, the procedure is identical to the one illustrated above.